SENSORS USED IN HEALTH CARE

SENSORS IN HEALTHCARE ARE DEVICE THAT DETECT AND MEASURE BIOLOGICAL,PHYSICAL, OR CHEMICAL CHANGES IN THE HUMAN BODY AND CONVERT THEM INTO SIGNALS FOR DIAGNOSIS, MONITORING, OR TREATMENT.

TYPES OF SENSORS:

- TEMPERATURE SENSORS

- PULSE OXIMETER SENSORS

- RESPIRATORY/BREATHING SENSORS

- BLOOD GLUCOSE SENSORS

TEMPERATURE SENSOR:

DEFINITION: Device that measure body temperature and convert it into an electrical signal.

WORKING PRINCIPLE: Work based on change in resistance, voltage, or current due to temperature variations.

ADVANTAGES:

- Fast and accurate measurement.

- Non-invasive(infrared) or simple to use.

- Portable and low-cost

DISADVANTAGES:

- Skin temperature may not always represent core body temperature.

- Can be affected by environment(air, sweat ,etc..).

TEMPERATURE SENSOR

PULSE OXIMETER SENSORS:

DEFINITION: Device that measure oxygen saturation(SpO2) and pulse rate.

WORKING PRINCIPLE: Use photoplethysmography(PPG)- red and infrared light are passed through a fingertip or earlobe, and difference in light absorptions by oxygenated vs deoxygenated hemoglobin are measured.

ADVANTAGES

- Non-invasive and painless.

- Provides real-time oxygen monitoring.

- Portable and easy to use.

DISADAVANTAGES

- Accuracy can be reduced by motion, nail polish, poor circulation, or bright light.

- Limited information(doesn’t measure CO2).

This Photo by Unknown Author is licensed under CC BY-SA

PULSE OXIMETER SENSOR

RESPIRATORY/BREATHING SENSORS:

DEFINITION: Sensors that detect and monitor breathing rate and patterns.

WORKING PRINCIPLE: Strain gauges detect chest / abdominal expansion. Thermistor detect temperature difference in inhaled / exhaled air. Capacitive sensor detect air flow or chest movement.

ADVANTAGES:

- Non-invasive continuous monitoring.

- Useful for sleep apnea and lung disease monitoring.

DISADVANTAGES:

- Accuracy affected by body movement.

- May require calibration.

- Some types can be uncomfortable(wearable belt).

BLOOD PRESSURE SENSOR:

DEFINITION: Device that measure the concentration of glucose in the blood, commonly used by diabetic patients.

WORKING PRINCIPLE: Electrochemical principle enzymes react with glucose producing electrons measured as electrical signals. Some newer sensors use optical / infrared methods.

ADVANTAGES:

- Provides critical data for diabetes management.

- Portable and gives quick results.

- Continuous glucose monitors reduce finger pricks.

DISADVANTAGES:

- Invasive(finger pricks or skin sensor)

- Sensor cost is high

- Accuracy may be affected by hydration, temperature, or pressure on the sensors.

CONCLUSION: HEALTHCARE SENSORS ARE FOUNDATIONAL TECHNOLOGY, PROVIDING REAL-TIME, CONTINOUS DATA FOR IMPROVED PATIENT MONITORING, EARLY DIAGNOSIS, AND PERSONALIZED TREATMENT, DRIVING ADVANCEMENTS THROUGH MINIATURIZATION, MULTIFUNCTIONALITY, AND AI INTEGRATION.